## 16F1503 Support Information

### #pragma config Usage

#### #pragma config <setting>=<named value>

For example:
// Brown-out Reset Enable: Brown-out Reset disabled
// Oscillator Selection Bits: ECM, External Clock, Medium Power Mode (0.5-4 MHz): device clock supplied to CLKIN pin
// MCLR Pin Function Select: MCLR/VPP pin function is digital input
// Watchdog Timer Enable: WDT disabled
// Flash Program Memory Code Protection: Program memory code protection is disabled
// Power-up Timer Enable: PWRT disabled
// Clock Out Enable: CLKOUT function is disabled. I/O or oscillator function on the CLKOUT pin
#pragma config BOREN = OFF, FOSC = ECM, MCLRE = OFF, WDTE = OFF, CP = OFF, PWRTE = OFF, CLKOUTEN = OFF

#### #pragma config <setting>=<literal constant>

For example:
// Brown-out Reset Enable: Brown-out Reset disabled
// Oscillator Selection Bits: ECM, External Clock, Medium Power Mode (0.5-4 MHz): device clock supplied to CLKIN pin
// MCLR Pin Function Select: MCLR/VPP pin function is digital input
// Watchdog Timer Enable: WDT disabled
// Flash Program Memory Code Protection: Program memory code protection is disabled
// Power-up Timer Enable: PWRT disabled
// Clock Out Enable: CLKOUT function is disabled. I/O or oscillator function on the CLKOUT pin
#pragma config BOREN = 0x0, FOSC = 0x2, MCLRE = 0x0, WDTE = 0x0, CP = 0x1, PWRTE = 0x1, CLKOUTEN = 0x1

#### #pragma config <register>=<literal constant>

For example: 
// Brown-out Reset Enable: Brown-out Reset disabled
// Oscillator Selection Bits: ECM, External Clock, Medium Power Mode (0.5-4 MHz): device clock supplied to CLKIN pin
// MCLR Pin Function Select: MCLR/VPP pin function is digital input
// Watchdog Timer Enable: WDT disabled
// Flash Program Memory Code Protection: Program memory code protection is disabled
// Power-up Timer Enable: PWRT disabled
// Clock Out Enable: CLKOUT function is disabled. I/O or oscillator function on the CLKOUT pin
#pragma config CONFIG1 = 0xF9A6

For example: 
// IDLOC @ 0x8000
#pragma config IDLOC0 = 0x3FFF

### #pragma config Settings

#### Register: CONFIG1 @ 0x8007

| BOREN = | Brown-out Reset Enable |
| --- | --- |
| OFF | Brown-out Reset disabled |
| ON | Brown-out Reset enabled |
| NSLEEP | Brown-out Reset enabled while running and disabled in Sleep |
| SBODEN | Brown-out Reset controlled by the SBOREN bit in the BORCON register |

| FOSC = | Oscillator Selection Bits |
| --- | --- |
| ECM | ECM, External Clock, Medium Power Mode (0.5-4 MHz): device clock supplied to CLKIN pin |
| ECH | ECH, External Clock, High Power Mode (4-20 MHz): device clock supplied to CLKIN pin |
| ECL | ECL, External Clock, Low Power Mode (0-0.5 MHz): device clock supplied to CLKIN pin |
| INTOSC | INTOSC oscillator: I/O function on CLKIN pin |

| MCLRE = | MCLR Pin Function Select |
| --- | --- |
| OFF | MCLR/VPP pin function is digital input |
| ON | MCLR/VPP pin function is MCLR |

| WDTE = | Watchdog Timer Enable |
| --- | --- |
| OFF | WDT disabled |
| ON | WDT enabled |
| NSLEEP | WDT enabled while running and disabled in Sleep |
| SWDTEN | WDT controlled by the SWDTEN bit in the WDTCON register |

| CP = | Flash Program Memory Code Protection |
| --- | --- |
| OFF | Program memory code protection is disabled |
| ON | Program memory code protection is enabled |

| PWRTE = | Power-up Timer Enable |
| --- | --- |
| OFF | PWRT disabled |
| ON | PWRT enabled |

| CLKOUTEN = | Clock Out Enable |
| --- | --- |
| OFF | CLKOUT function is disabled. I/O or oscillator function on the CLKOUT pin |
| ON | CLKOUT function is enabled on the CLKOUT pin |

#### Register: CONFIG2 @ 0x8008

| LPBOR = | Low-Power Brown Out Reset |
| --- | --- |
| OFF | Low-Power BOR is disabled |
| ON | Low-Power BOR is enabled |

| WRT = | Flash Memory Self-Write Protection |
| --- | --- |
| OFF | Write protection off |
| BOOT | 000h to 1FFh write protected, 200h to 1FFFh may be modified by EECON control |
| HALF | 000h to FFFh write protected, 1000h to 1FFFh may be modified by EECON control |
| ALL | 000h to 1FFFh write protected, no addresses may be modified by EECON control |

| STVREN = | Stack Overflow/Underflow Reset Enable |
| --- | --- |
| OFF | Stack Overflow or Underflow will not cause a Reset |
| ON | Stack Overflow or Underflow will cause a Reset |

| BORV = | Brown-out Reset Voltage Selection |
| --- | --- |
| LO | Brown-out Reset Voltage (Vbor), low trip point selected. |
| HI | Brown-out Reset Voltage (Vbor), high trip point selected. |

| LVP = | Low-Voltage Programming Enable |
| --- | --- |
| OFF | High-voltage on MCLR/VPP must be used for programming |
| ON | Low-voltage programming enabled |

#### Register: IDLOC0 @ 0x8000

#### Register: IDLOC1 @ 0x8001

#### Register: IDLOC2 @ 0x8002

#### Register: IDLOC3 @ 0x8003