## 16F1828 Support Information

### #pragma config Settings

#### Register: CONFIG1 @ 0x8007

| CPD = | Data Memory Code Protection |
| --- | --- |
| OFF | Data memory code protection is disabled |
| ON | Data memory code protection is enabled |

| BOREN = | Brown-out Reset Enable |
| --- | --- |
| OFF | Brown-out Reset disabled |
| ON | Brown-out Reset enabled |
| NSLEEP | Brown-out Reset enabled while running and disabled in Sleep |
| SBODEN | Brown-out Reset controlled by the SBOREN bit in the BORCON register |

| IESO = | Internal/External Switchover |
| --- | --- |
| OFF | Internal/External Switchover mode is disabled |
| ON | Internal/External Switchover mode is enabled |

| FOSC = | Oscillator Selection |
| --- | --- |
| ECM | ECM, External Clock, Medium Power Mode (0.5-4 MHz): device clock supplied to CLKIN pin |
| XT | XT Oscillator, Crystal/resonator connected between OSC1 and OSC2 pins |
| LP | LP Oscillator, Low-power crystal connected between OSC1 and OSC2 pins |
| EXTRC | EXTRC oscillator: External RC circuit connected to CLKIN pin |
| ECH | ECH, External Clock, High Power Mode (4-32 MHz): device clock supplied to CLKIN pin |
| ECL | ECL, External Clock, Low Power Mode (0-0.5 MHz): device clock supplied to CLKIN pin |
| INTOSC | INTOSC oscillator: I/O function on CLKIN pin |
| HS | HS Oscillator, High-speed crystal/resonator connected between OSC1 and OSC2 pins |

| FCMEN = | Fail-Safe Clock Monitor Enable |
| --- | --- |
| OFF | Fail-Safe Clock Monitor is disabled |
| ON | Fail-Safe Clock Monitor is enabled |

| MCLRE = | MCLR Pin Function Select |
| --- | --- |
| OFF | MCLR/VPP pin function is digital input |
| ON | MCLR/VPP pin function is MCLR |

| WDTE = | Watchdog Timer Enable |
| --- | --- |
| OFF | WDT disabled |
| ON | WDT enabled |
| NSLEEP | WDT enabled while running and disabled in Sleep |
| SWDTEN | WDT controlled by the SWDTEN bit in the WDTCON register |

| CP = | Flash Program Memory Code Protection |
| --- | --- |
| OFF | Program memory code protection is disabled |
| ON | Program memory code protection is enabled |

| PWRTE = | Power-up Timer Enable |
| --- | --- |
| OFF | PWRT disabled |
| ON | PWRT enabled |

| CLKOUTEN = | Clock Out Enable |
| --- | --- |
| OFF | CLKOUT function is disabled. I/O or oscillator function on the CLKOUT pin |
| ON | CLKOUT function is enabled on the CLKOUT pin |

#### Register: CONFIG2 @ 0x8008

| PLLEN = | PLL Enable |
| --- | --- |
| OFF | 4x PLL disabled |
| ON | 4x PLL enabled |

| WRT = | Flash Memory Self-Write Protection |
| --- | --- |
| OFF | Write protection off |
| BOOT | 000h to 1FFh write protected, 200h to FFFh may be modified by EECON control |
| HALF | 000h to 7FFh write protected, 800h to FFFh may be modified by EECON control |
| ALL | 000h to FFFh write protected, no addresses may be modified by EECON control |

| STVREN = | Stack Overflow/Underflow Reset Enable |
| --- | --- |
| OFF | Stack Overflow or Underflow will not cause a Reset |
| ON | Stack Overflow or Underflow will cause a Reset |

| BORV = | Brown-out Reset Voltage Selection |
| --- | --- |
| LO | Brown-out Reset Voltage (Vbor), low trip point selected. |
| HI | Brown-out Reset Voltage (Vbor), high trip point selected. |

| LVP = | Low-Voltage Programming Enable |
| --- | --- |
| OFF | High-voltage on MCLR/VPP must be used for programming |
| ON | Low-voltage programming enabled |

#### Register: IDLOC0 @ 0x8000

#### Register: IDLOC1 @ 0x8001

#### Register: IDLOC2 @ 0x8002

#### Register: IDLOC3 @ 0x8003