HIB Diagnostic.   Gottardo Marco

The diagnostic data for carrying out this task are the plasma current, the ring voltage Vloop, the electron density, the initial current of HIB (I0+), HIB residual current on the detector and HIB interaction current (Ij2+) for every one of the 12 detector sectors, over all the cycle period.

The sampling frequency is set to 2 kHz.

The goal is to calculate the simplified neσ profile in one cycle of a plasma discharge, using data provided by the Heavy Ion beam Diagnostic.

Plasma current on a discharge.

In the time between 35 and 55ms the phenomenon presents an oscillation but without exceeding 25% of the maximum value.

Before 35ms we neglect each value because the plasma current is forming, while after 55ms the plasma ramp down brings the plasma current to zero.

The neσ profile will depend slightly from this parameter (this explains why we are considering only the time period 35-55 ms).

Using the formula:

neσ12rj=Ij(det)2+2Ij+dl ,

where the primary current Ij+ is:

Ij+=I0+-L=0j-112IL(det)2+ ,

it is possible to retrieve neσ for each sector, being IL(det)2+ secondary current measured at different radial plasma positions (i.e. detector sectors) and dl=1cm the width of the sector.

In the plot the neσ profile for each sector can be seen, along the entire cycle period.

3D neσ profile. Spatial and temporal

The previous figure shows both temporal and spatial inhomogeneity of neσ.

It is necessary to introduce other parameters to find an explanation to the lack of homogeneity in neσ, such as the temperature of the plasma.

It seems insufficient to analyse the plasma current in the range considered.

The spritzer resistivity ρ, gives information about plasma temperature, as shown by the formula:

ρ∝1Te32 .

Using the available data on Vloop and plasma current, is possible to estimate ρ:

ρ=VloopIp .

Considering neσ integral over the detector sectors is possible to plot this quantity in function of time allowing comparison with plasma temperature (obtained from the Spitzer resistivity) and electron density.

The data is in a database populated by interferometry measures.

The neσ and temperature plots show how the two quantities are varying with the same phase

The electron density is not correlated with neσ and temperature fluctuations, the neσ Inhomogeneity in time is this way justified.  As for spatial inhomogeneity though, we have not enough information and different diagnostics are necessary.