Langmuir probes data analysis.

Gottardo Marco

Objective of the task is to analyze the available data collected by spectral analysis and other statistical tools. Signals are collected by 12 Langmuir sensors  , Installed on the ISTTOK machine.

The provided data refers to the three ISTTOK pulses listed below.

- Shot 16572 - probe at r = 75 mm measuring Vf

- Shot 16573 - probe at r = 85 mm measuring Vf

- Shot 16574 - probe at r = 85 mm measuring Isat

Langmuir probes have  8-pins, 2 mm distant to each other.

One of the eight available pins has been found not working. Data from this are inconsistent.

For each the position (r) in the plasma was provided, since they can be moved along the radius of the torus poloidal plasma.

All calculations and the images are obtained using Matlab.

The interest of the analysis is to highlight the fluctuations eliminating the average value of the signal.

A simple subtraction allows us to achieve the purpose.

The red line shows the value of the trend. the fluctuations is shown as black peaks.

The MatLab function called pdf allows us to calculate the probability density function and plot the diagram.

The probability density function of the 5th pin of probe 16573 (Vf) and 16574 (Isat), which are at the same position.

On the same data we can plot the next images that show the statistical analysis of signals detected by the sensors.

The following figures are showing histograms, mean values and standard deviation for all the signals (please note that the sensor number 6 is inoperative.).

The following  plot reveal that the average values are considerably relaxed as we expected, the standard deviation for the pulse 16572 is rather remarkable suggesting that something is irregular.

We proceed by employing the function 'spectrogram' in order to highlight the content of the signal in frequency.

Spectrograms plotted for pulse 16572 and 16573 show the same analysis in Vf in different radial positions of the probes.

Perturbation is detected between the frequency of 5 kHz up to 30 kHz for the duration of approximately 20ms. (from t=10ms to t=30ms).

The coherence analysis, comparing the signals received from the different pins of the probe to identify the presence of coherent structures.

Plot of the signals, obtained through the MatLab functions 'mscohere' and 'cpsd', compared to specific phases for the operating pin 5 of the probes.

Graphs show a coherent structure centered between frequencies of 5 kHz to ~ ~ 30kHz. (for not 16572)

For the other shot, no coherent signals are observed. This result is in agreement with what was observed previously from the spectrogram.

Further investigation on the cross correlation in the impulse 16572 are performed using the function xcorr.

The results show us how six signals are similar also shifting the position in space and time in the ISSTOK.

From the cross correlation it is possible to get informations about correlation length, characteristic times and propagation speed of the turbulence.

|  |  |  | Correlation lengths [um] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | Broken pin | 7 | 8 |
| 1 | # | # | 180 | 176 | 82 | / | 78 | 52 |
| 2 | # | # | # | 104 | 180 | / | 84 | 82 |
| 3 | -192 | # | # | 106 | 208 | / | 180 | 90 |
| 4 | -168 | -102 | -102 | # | # | / | 176 | 176 |
| 5 | -72 | -156 | -164 | # | # | / | 172 | 172 |
| Broken pin | / | / | / | / | / | / | / | / |
| 7 | -60 | -64 | -126 | -144 | -152 | / | # | 196 |
| 8 | -38.6 | -62 | -66 | -144 | -152 | / | -200 | # |

Pulse 16573

|  |  |  | Propagation speeds [m/s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | Broken pin | 7 | 8 |
| 1 | # | # | 4 | 4 | 2 | / | 2 | 1.3 |
| 2 | # | # | # | 4 | 4 | / | 2 | 2 |
| 3 | -4 | # | # | 4 | 4 | / | 4 | 2 |
| 4 | -4 | -4 | -4 | # | # | / | 4 | 4 |
| 5 | -2 | -4 | -4 | # | # | / | 4 | 4 |
| Broken pin | / | / | / | / | / | / | / | / |
| 7 | -2 | -2 | -4 | -4 | -4 | / | # | 4 |
| 8 | -1.3 | -2 | -2 | -4 | -4 | / | -4 | # |

|  |  |  | Characteristic time [us] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | Broken pin | 7 | 8 |
| 1 | 43 | 45 | 45 | 44 | 41 | / | 39 | 39 |
| 2 | 46 | 25.5 | 25.5 | 26 | 45 | / | 42 | 41 |
| 3 | 48 | 26 | 26 | 26.5 | 52 | / | 45 | 45 |
| 4 | 42 | 25.5 | 25.5 | 26 | 49 | / | 44 | 44 |
| 5 | 36 | 39 | 41 | 43 | 43 | / | 43 | 43 |
| Broken pin | / | / | / | / | / | / | / | / |
| 7 | 30 | 32 | 34 | 36 | 38 | / | 26.5 | 49 |
| 8 | 29 | 31 | 33 | 36 | 38 | / | 50 | 27 |

The propagation speed has been computed investigating the time distance between peak centres on the cross correlation graph and considering the 2mm of distance between two probe pins. Characteristic time is the time distance.

The characteristic length is obtained multiplying the previous informations.