Plasma spectroscopy lab

Gottardo Marco

The assigned task is to assign a particular element to the lines shown in the spectroscopic diagram  given us.

The spectrum allocated is related to an Argon plasma [Ar] at a pressure of 0,36mbar at 200w of power.

The graph window is placed in a wavelength between 86 and 124 nm in a far ultra violet.

1

2

3

4

5

6

7

8

9

11

10

12

13

14

The data is available on database NIST Atomic Spectra Database that allows, by comparison, to associate the rows at specific wavelengths to detected elements.

Each line has been associated with an numbered arrow.

In the following table we inserted the elements corresponding to the line indicated by the numbered arrow.

The small peaks around the lines introduce a significant contribution which is inserted in the table.

The table is more populated by the small peaks in the shorter wavelength side of the graph.

The impure elements detected, oxygen, nitrogen, hydrogen, indicate a probable remanence of air into the plasma chamber.

Table of detected elements.

| # | nm | element |
| --- | --- | --- |
| 1 | 86.68 / 87,1 | ArI/ArIII |
| 2 | 87.87 / 86,60 | ArIII/ArI |
| 3 | 88.31 / 87.99 | ArIII/ArI |
| 4 | 89.43 | ArI |
| 5 | 91.97 | ArII |
| 6 | 93.20 | ArII |
| 7 | 97.25 | HI |
| 8 | 98.87 / 98.97 | OI/NIII |
| 9 | 102.57 | HI |
| 10 | 104.82 | ArI |
| 11 | 106.67 | ArI |
| 12 | 113.49 | NI |
| 13 | 119.95 | NI |
| 14 | 121.57 | HI |

The second task was to measure the atomic temperature applying the Doppler broadening technique. We were provided with some experimental data about the spectroscopic measurement of the Hβ line  (486,1 nm) of a plasma experiment.  Using the software ORIGIN as tool for this study, we fitted the experimental data with a vogit profile.

From the vogit fit we retrieve informations about the Gaussian (∆λG) and Lorentian  ∆λLcontribute to the FWHM of the curve and the centre of the line:

∆λG=0.025731 nm

∆λL~ 0 nm

λHβ=486.1 nm

It came out that the Lorenzian contribution is so small that we can easily assume that the cure has a Gaussian shape.

The Doppler broadening can be now calculated, as we now that the instrumental contribution is ∆λi=0.004 nm :

Doppler broadening allows us to calculate the atomic temperature of the considered element by man of the formula:

Considering that , the H temperature inside the plasma is 5333 °K.